KOVURU JASWANTH 192412254)

Experiment: 7:Analog Beamforming Analysis Using MATLAB

Aim

To analyze the performance of analog beamforming by studying beamforming gain, beam width,

and main lobe direction using MATLAB.

Objectives

1. To analyze the beamforming gain achieved by varying the number of antenna elements in

an analog beamforming system using MATLAB.

2. To compare the beam width of an antenna array for different beamforming configurations

using MATLAB.

3. To analyze the main lobe direction by steering the antenna array beam to different angles

using MATLAB.

Software Required

• MATLAB

Theory

Analog beamforming is a beam steering technique in which a single RF chain drives multiple

antenna elements through adjustable phase shifters. By controlling the phase of the transmitted or

received signals, the antenna array forms a directional beam toward the intended user, thereby

improving signal strength, coverage, and spectral efficiency. Analog beamforming is widely

employed in millimeter-wave (mmWave) communication systems due to its reduced hardware

complexity and lower power consumption.

Objective – 1

Analyze Beamforming Gain (dB)

Generate beamforming gain values for different antenna array sizes using MATLAB. Compare

how increasing the number of antenna elements improves the beamforming gain and enhances

signal strength.

MATLAB Code:

clc;

clear;

close all;

antennas = [2 4 8 16 32]; % Number of Antenna Elements

gain = 10*log10(antennas); % Beamforming Gain (dB)

figure

bar(antennas, gain)

grid on

xlabel('Number of Antenna Elements')

ylabel('Beamforming Gain (dB)')

title('Beamforming Gain for Different Antenna Array Sizes')

Expected Output

• Bar graph showing beamforming gain for different antenna array sizes.

Objective – 2

Compare Beam Width (Degrees)

Analyze the beam width corresponding to different antenna array sizes using MATLAB. Observe

how increasing the number of antenna elements results in a narrower beam, thereby improving

spatial resolution.

MATLAB Code:

clc;

clear;

close all;

antennas = [2 4 8 16 32]; % Number of Antenna Elements

beamwidth = [90 45 22.5 11.25 5.625]; % Beam Width (degrees)

figure

plot(antennas, beamwidth,'o-','LineWidth',2)

grid on

xlabel('Number of Antenna Elements')

ylabel('Beam Width (Degrees)')

title('Beam Width for Different Antenna Array Sizes')

Expected Output

• Line graph showing beam width versus antenna array size.

Objective – 3

Analyze Main Lobe Direction (Degrees)

Simulate beam steering by varying the steering angle of the antenna array using MATLAB.

Observe the change in the main lobe direction and understand how analog beamforming directs

energy toward the intended user.

MATLAB Code:

clc;

clear;

close all;

theta = [-60 -30 0 30 60]; % Main Lobe Direction (degrees)

gain = [1 1 1 1 1]; % Constant normalized gain

figure

polarplot(deg2rad(theta), gain, 'o', 'LineWidth', 2)

title('Main Lobe Direction in Analog Beamforming')

Expected Output

• Polar plot or beam pattern showing the main lobe steered to different angles.

Result

The MATLAB analysis successfully demonstrated the characteristics of analog beamforming. The

results showed that increasing the number of antenna elements improves beamforming gain and

reduces beam width, while phase adjustment enables the main lobe to be steered toward different

directions.